# GAS CYLINDER WEIGHT MONITOR

# MASTER DESIGN DOCUMENT (MDD)

## Version 2.0

## PART 4 — SYSTEM ARCHITECTURE, DATA FLOW & OPERATIONAL MODEL

# 54. INTRODUCTION

Previous sections answered:

Part 1: Why the product exists.

Part 2: How gas estimation works.

Part 3: How trustworthy weight is produced.

This section answers:

How the entire system operates as a coordinated whole.

# 55. ARCHITECTURAL PHILOSOPHY

The system intentionally separates:

Measurement

from

Understanding

## Why

The responsibilities are fundamentally different.

Measurement requires:

- Determinism

- Timing accuracy

- Hardware interaction

Understanding requires:

- Storage

- Analytics

- Prediction

- User interfaces

Combining them increases complexity.

Separating them simplifies reasoning.

# 56. SYSTEM ARCHITECTURE

High-Level Architecture

Load Cell(s)
      ↓
HX711
      ↓
ESP32-C3
      ↓
Transport Layer
      ↓
UNO Q Hub
      ↓
Storage Layer
      ↓
Analytics Layer
      ↓
Prediction Layer
      ↓
Presentation Layer

# 57. ARCHITECTURAL RESPONSIBILITIES

The architecture intentionally creates two independent systems.

## Sensor System

Mission:

Determine weight.

Components:

Load Cell

HX711

ESP32-C3

Question Answered:

“What is the weight right now?”

## Intelligence System

Mission:

Determine meaning.

Components:

UNO Q

Storage

Analytics

Prediction

UI

Question Answered:

“What should the user do?”

# 58. SENSOR NODE DESIGN

The ESP32-C3 becomes a dedicated sensor appliance.

## Inputs

HX711 readings

Configuration

Time synchronization

Commands from hub

## Outputs

Weight readings

Events

Health reports

Diagnostics

## Responsibilities

Read sensor

Validate data

Filter corruption

Detect events

Transmit results

## Non-Responsibilities

Prediction

Long-term storage

Business logic

Complex analytics

Cloud management

# 59. HUB DESIGN

The UNO Q becomes the intelligence hub.

## Inputs

Sensor data

Historical data

Configuration

User actions

## Outputs

Insights

Predictions

Reports

Dashboards

Alerts

## Responsibilities

Store history

Analyze usage

Predict depletion

Serve interfaces

Coordinate devices

# 60. WHY THE HUB EXISTS

Without a hub:

The ESP32 would need to perform:

Storage

Analytics

Prediction

User interaction

This would increase:

Complexity

Power consumption

Maintenance burden

The hub centralizes intelligence.

The node remains simple.

# 61. DATA FLOW

The system follows a pipeline.

Measurement Flow

Load Cell
↓
HX711
↓
ESP32
↓
Validated Weight

Transport Flow

Validated Weight
↓
Network Message
↓
UNO Q

Intelligence Flow

Weight
↓
Gas Estimate
↓
Analytics
↓
Prediction

Presentation Flow

Prediction
↓
Dashboard
↓
User

# 62. COMMAND FLOW

Data does not only move upward.

Commands also move downward.

Example

User requests current status.

Flow

User
↓
Hub
↓
ESP32
↓
Fresh Reading
↓
Hub
↓
User

This is called a Pull Request.

# 63. EVENT FLOW

Some information originates from the node.

Example

Cylinder Installed

Flow

ESP32
↓
Event Detection
↓
Hub
↓
Storage
↓
Notification

This is called Event Push.

# 64. HYBRID COMMUNICATION MODEL

The project uses both:

Pull

and

Push

## Pull

User requests information.

System responds.

## Push

System detects something important.

System informs the user.

Both are necessary.

# 65. SAMPLING ARCHITECTURE

One of the most important design decisions.

The project separates:

Reading

from

Recording

Many systems incorrectly combine them.

# 66. INTERNAL READS

Purpose:

Maintain awareness.

Characteristics

Fast

Frequent

Not Stored

Uses

Event detection

Pull requests

Health monitoring

Question Answered

“What is happening right now?”

# 67. HEARTBEAT RECORDS

Purpose:

Create history.

Characteristics

Slow

Regular

Stored

Current Design

15-minute interval

Uses

Analytics

Prediction

Trend analysis

Leak detection

Question Answered

“What happened over time?”

# 68. EVENT RECORDS

Purpose:

Capture significant moments.

Examples

Install

Removal

Session start

Session end

Low gas

Characteristics

Sparse

High value

Context-rich

Question Answered

“What important thing happened?”

# 69. WHY THREE DATA STREAMS EXIST

Internal Reads

Answer: Now

Heartbeat Records

Answer: History

Event Records

Answer: Meaning

Combining them creates confusion.

Separating them creates clarity.

# 70. STATE MACHINES

The system behaves differently depending on state.

State machines formalize this behavior.

# 71. NODE STATE MACHINE

States

BOOT
↓
INITIALISING
↓
READY
↓
MEASURING
↓
DEGRADED
↓
FAILED

BOOT

Power-up.

INITIALISING

Loading configuration.

Checking hardware.

READY

Waiting for activity.

MEASURING

Actively collecting data.

DEGRADED

Reduced confidence.

Higher noise.

Partial functionality.

FAILED

Measurement cannot be trusted.

# 72. CYLINDER STATE MACHINE

States

UNKNOWN
↓
FULL
↓
ACTIVE
↓
LOW
↓
EMPTY
↓
REMOVED
↓
REPLACED

UNKNOWN

No reliable anchor.

FULL

Fresh install detected.

ACTIVE

Normal usage.

LOW

Approaching depletion.

EMPTY

Gas effectively exhausted.

REMOVED

Cylinder removed.

REPLACED

New cylinder installed.

# 73. MEASUREMENT STATE MACHINE

States

UNSTABLE
↓
SETTLING
↓
STABLE
↓
CAPTURED

UNSTABLE

Weight changing.

SETTLING

Movement stopped.

Verifying stability.

STABLE

Measurement trusted.

CAPTURED

Stored or transmitted.

# 74. HEALTH STATE MACHINE

States

GOOD
↓
WARNING
↓
DEGRADED
↓
FAILED

GOOD

Expected operation.

WARNING

Potential issue emerging.

DEGRADED

Accuracy affected.

FAILED

Service unavailable.

# 75. FAILURE PHILOSOPHY

Systems fail.

This is expected.

The design goal is not:

Prevent all failures.

The design goal is:

Fail predictably.

Fail visibly.

Recover automatically.

# 76. FAILURE CATEGORIES

Sensor Failure

Transport Failure

Storage Failure

Calibration Failure

Prediction Failure

Each category requires different recovery behavior.

# 77. TRANSPORT FAILURE

Example

WiFi unavailable.

Node behavior:

Continue measuring.

Buffer data.

Retry later.

Hub behavior:

Mark node offline.

Preserve history.

# 78. SENSOR FAILURE

Example

HX711 stops responding.

System should:

Detect

Report

Isolate

Recover if possible

Never silently continue.

# 79. STORAGE FAILURE

Example

Database unavailable.

System should:

Protect incoming data.

Attempt recovery.

Log diagnostics.

Measurement should continue.

# 80. RECOVERY PHILOSOPHY

Every failure should have:

Detection

Classification

Response

Recovery

Verification

A recovered system must prove it is healthy again.

# 81. CONFIGURATION PHILOSOPHY

Configuration should contain:

Node identity

Calibration values

Thresholds

Network settings

Configuration should not contain:

Hidden logic

Business rules

Hardcoded assumptions

# 82. FUTURE EXPANSION

The architecture intentionally supports growth.

Current

One Node

One Cylinder

One Hub

Future

Multiple Nodes

Multiple Cylinders

Multiple Hubs

Cloud Integration

No architectural redesign should be required.

# 83. SYSTEM ARCHITECTURE SUMMARY

The architecture separates:

Measurement

from

Intelligence

and

Data Collection

from

Data Understanding.

The ESP32 answers:

“What is the weight?”

The UNO Q answers:

“What does the weight mean?”

This separation creates a system that is easier to reason about, easier to test, easier to expand, and more resilient to future changes.

END OF PART 4