# GAS CYLINDER WEIGHT MONITOR

# MASTER DESIGN DOCUMENT (MDD)

## Version 2.0

## Status: Active Design

## Date: 2026-06-03

# DOCUMENT PURPOSE

This document is the authoritative design reference for the Gas Cylinder Weight Monitor project.

It consolidates:

- Product vision

- Domain knowledge

- Architecture decisions

- Mathematical models

- Calibration philosophy

- Measurement theory

- Analytics strategy

- Prediction strategy

- Engineering principles

- Roadmap planning

This document is intended to survive hardware changes, software rewrites, transport changes, and implementation pivots.

The purpose is to preserve understanding.

# 1. EXECUTIVE SUMMARY

## Project Goal

Create a smart LPG monitoring system that continuously measures the weight of a gas cylinder, determines remaining LPG, learns household consumption behavior, predicts future depletion, and proactively warns users before gas runs out.

The long-term objective is not weight measurement.

The long-term objective is decision support.

The scale is merely the sensing mechanism.

The product value comes from:

Weight → Gas Remaining → Consumption Understanding → Prediction → Action

## Core Problem

Households typically discover an empty cylinder only after attempting to cook.

Existing methods are unreliable:

- Lifting the cylinder by hand

- Shaking the cylinder

- Guessing based on time elapsed

- Waiting until gas stops flowing

None provide:

- Remaining gas

- Daily consumption

- Predicted depletion date

- Usage patterns

The proposed system solves these problems through continuous weight measurement and intelligent interpretation.

## Product Philosophy

The project is built around several principles:

### Measurement before prediction

Predictions are worthless if measurements are wrong.

### Understanding before implementation

Every major design decision must be explainable from first principles.

### Small → Verify → Compound

Complex systems emerge from validated building blocks.

### No magic constants

Values should be measured, derived, or configurable.

### Continuous validation

Nothing is trusted forever.

Everything can drift.

Everything must be revalidated.

# 2. PRODUCT VISION

## Vision Statement

A household should always know:

- How much gas remains.

- How quickly it is being consumed.

- When it will likely run out.

- Whether current usage is normal.

without manually checking the cylinder.

## Immediate Benefits

### Remaining Gas

Users can view:

- Remaining gas in grams

- Remaining gas percentage

### Consumption Awareness

Users can understand:

- Daily consumption

- Weekly consumption

- Monthly trends

### Run-Out Prediction

Users receive:

- Days remaining

- Expected depletion date

- Early warning alerts

### Historical Understanding

The system becomes a long-term record of LPG usage.

Patterns emerge naturally:

- Cooking frequency

- Seasonal changes

- Household behavior shifts

# 3. PRODUCT EVOLUTION ROADMAP

The project intentionally separates the complete vision from the first shipping version.

## V1 — Known Full Cylinder

### Assumption

The user installs the system when a fresh cylinder is delivered.

This assumption removes the hardest cold-start problem.

### Capabilities

- Exact steel derivation

- Exact gas remaining

- Exact percentage

- Burn-rate tracking

- Consumption analytics

- Days-remaining prediction

### Goal

Deliver useful value as quickly as possible.

## V2 — Assisted Partial Cylinder

### Scenario

A cylinder is already in use.

The system did not witness installation.

### Additional Inputs

The user may provide:

- Stamped tare weight

- Cylinder type

- Other known information

### Result

The system can bootstrap gas estimates immediately.

## V3 — Fully Autonomous

### Scenario

Unknown cylinder.

Unknown state.

No user assistance.

### Strategy

Use:

- Interval estimation

- Refill detection

- Empty-floor detection

- Self-healing anchor logic

### Goal

Recover automatically over time.

# 4. SYSTEM OVERVIEW

## High-Level Architecture

Load Cell(s) ↓ HX711 ↓ ESP32-C3 Sensor Node ↓ WiFi ↓ UNO Q Hub ↓ Storage Analytics Prediction Presentation

## Why The Architecture Changed

Earlier designs used the UNO Q STM32 MCU as the sensor processor.

The architecture evolved into:

ESP32-C3 = Sensor Node

UNO Q = Hub

This separation creates clearer responsibilities.

## ESP32-C3 Mission

The ESP32 exists to answer one question:

“What is the weight right now?”

Responsibilities:

- Read HX711

- Filter noise

- Detect invalid readings

- Convert raw values

- Detect events

- Report measurements

The ESP32 does not own business logic.

## UNO Q Mission

The UNO Q exists to answer:

“What does the weight mean?”

Responsibilities:

- Store measurements

- Learn behavior

- Predict future consumption

- Present information

- Serve applications

The UNO Q owns intelligence.

# 5. FUNDAMENTAL DOMAIN MODEL

This section contains the most important idea in the entire project.

## What The Scale Measures

The scale measures:

Gross Weight

Nothing else.

The scale does not measure:

- Gas

- Percentage

- Burn rate

- Days remaining

Those are derived quantities.

## The Fundamental Equation

Gross Weight = Steel Weight + Gas Weight

G = S + g

Where:

G = gross weight S = steel weight g = gas weight

## Why This Matters

The scale provides:

G

The user wants:

g

But:

G = S + g

contains two unknowns.

Therefore:

A single measurement cannot uniquely determine remaining gas.

This is a mathematical limitation.

Not a software limitation.

Not a sensor limitation.

Not an AI limitation.

The missing information must come from somewhere else.

# 6. THE CENTRAL INSIGHT

The entire project revolves around acquiring the missing equation.

Without it:

Remaining gas cannot be known.

With it:

The problem becomes trivial.

The rest of this document explains how the system acquires that missing information through anchor events, classification, and observation.

END OF PART 1