# GAS CYLINDER WEIGHT MONITOR

# MASTER DESIGN DOCUMENT (MDD)

## Version 2.0

## PART 2 — LPG DOMAIN MODEL & GAS ESTIMATION THEORY

# 7. LPG DOMAIN KNOWLEDGE

Before building software, the physical reality of LPG cylinders must be understood.

The system exists in the physical world.

Therefore:

The physics and regulations governing LPG cylinders are more important than any algorithm.

## Domestic LPG In India

The project initially targets Indian domestic LPG cylinders.

The most common domestic cylinder contains:

Net LPG = 14.2 kg

This quantity is tightly regulated.

The exact fill may vary slightly due to permitted tolerance, but the nominal capacity remains fixed.

## Commercial LPG

Commercial cylinders exist in other capacities.

Examples include:

- 19 kg

- 47.5 kg

- 425 kg

These capacities are discrete and well separated.

This separation becomes important later for classification.

## Important Observation

Gas quantity is standardized.

Steel quantity is not.

This asymmetry drives the entire architecture.

# 8. THE STEEL PROBLEM

Every LPG cylinder consists of:

Steel Shell

- LPG Contents

The steel shell remains on the scale throughout the life of the cylinder.

Only LPG leaves.

## Example

Full Cylinder

Steel = 15.3 kg

Gas = 14.2 kg

Gross = 29.5 kg

After Usage

Steel = 15.3 kg

Gas = 6.0 kg

Gross = 21.3 kg

Empty Cylinder

Steel = 15.3 kg

Gas = 0 kg

Gross = 15.3 kg

## Consequence

The scale never directly sees gas.

It always sees:

Steel + Gas

This is the fundamental challenge.

# 9. THE 52% TRAP

Many gas-monitor products make a conceptual mistake.

They assume:

percentage = current_weight / full_weight

## Example

Full:

29.5 kg

Empty:

15.3 kg

Incorrect Calculation:

15.3 / 29.5

≈ 52%

Result:

An empty cylinder appears half full.

This is catastrophic.

A user relying on such a gauge will run out unexpectedly.

## Design Rule

Any percentage calculation that does not explicitly remove steel weight is invalid.

This rule is non-negotiable.

# 10. HONEST GAS PERCENTAGE

The correct approach is:

Gas Remaining = Gross − Steel

Percentage = Gas Remaining / Capacity

## Domestic Example

Capacity = 14.2 kg

Steel = 15.3 kg

Gross = 21.3 kg

Gas Remaining:

21.3 − 15.3

= 6.0 kg

Percentage:

6.0 / 14.2

≈ 42%

## Interpretation

The gauge now correctly represents fuel.

Not total mass.

# 11. WHY BRAND DOES NOT MATTER

Early designs considered identifying:

- Indane

- HP

- Bharat Gas

- Others

This approach was ultimately rejected.

## Reason

Brand differences primarily affect:

Steel Weight

They do not significantly affect:

Gas Capacity

## What Users Care About

Users care about:

Gas Remaining

Not:

Cylinder Manufacturer

## Better Strategy

Measure steel directly.

Ignore brand.

Brand becomes unnecessary.

# 12. CAPACITY CLASSIFICATION

While steel varies continuously,

Capacity varies discretely.

This distinction is important.

## Example

Approximate Full Weights

Domestic 14.2 kg:

≈ 29–31 kg

Commercial 19 kg:

≈ 34–36 kg

47.5 kg:

Much larger

## Consequence

Capacity can often be classified from gross weight bands.

Steel cannot.

## Engineering Rule

Capacity should be classified.

Steel should be measured.

Never assume steel.

# 13. THE UNDERDETERMINED SYSTEM

This is the deepest concept in the project.

Suppose:

Gross = 22 kg

Question:

How much gas remains?

Possible Answer 1

Steel = 15.0 kg

Gas = 7.0 kg

Possible Answer 2

Steel = 15.5 kg

Gas = 6.5 kg

Possible Answer 3

Steel = 15.8 kg

Gas = 6.2 kg

All satisfy:

Gross = Steel + Gas

## Consequence

The problem has multiple valid solutions.

The system lacks sufficient information.

## Important Principle

No algorithm can recover information that does not exist.

The limitation is informational.

Not computational.

# 14. ANCHOR THEORY

The system needs a second equation.

That second equation arrives through anchor events.

## Definition

An Anchor Event is any moment where gas quantity becomes known.

Once gas is known,

Steel can be solved.

# 15. FULL INSTALL ANCHOR

Primary Anchor

Preferred Method

## Observation

A freshly delivered cylinder is full.

Therefore:

Gas = Capacity

## Equation

Gross Install = Steel + Capacity

Rearranging:

Steel = Gross Install − Capacity

## Example

Install Gross = 29.7 kg

Capacity = 14.2 kg

Steel =

29.7 − 14.2

= 15.5 kg

## Advantages

Automatic

No user input

No brand database

No waiting

Works every refill cycle

## Design Decision

Full Install Anchor is the primary calibration mechanism.

# 16. EMPTY FLOOR ANCHOR

Secondary Anchor

Cross-check Method

## Observation

An empty cylinder contains:

Gas ≈ 0

Therefore:

Gross ≈ Steel

## Advantages

Direct measurement

Very accurate

## Limitations

Not guaranteed.

Many users replace cylinders before reaching true empty.

## Design Decision

Use as verification.

Never depend on it.

# 17. CYLINDER LIFE CYCLE

Every cylinder naturally moves through states.

UNKNOWN ↓ FULL ↓ ACTIVE ↓ LOW ↓ EMPTY ↓ REMOVED ↓ REPLACED

## Importance

Every transition creates information.

The system should exploit these transitions.

# 18. PARTIAL CYLINDER COLD START

This is the hardest case.

Scenario:

System powers on.

Cylinder already exists.

No installation observed.

No removal observed.

No history exists.

Available Information:

Gross Weight

Only.

Unknown:

Steel

Gas

Result:

Exact gas quantity cannot be determined.

# 19. INTERVAL ESTIMATION

Although exact gas is impossible,

Bounds are possible.

Example

Gross = 22 kg

Steel Range:

14.5 – 15.8 kg

Gas Minimum:

22 − 15.8

= 6.2 kg

Gas Maximum:

22 − 14.5

= 7.5 kg

Result

Gas ∈ [6.2, 7.5]

Percentage

44% – 53%

## Interpretation

The system cannot know the exact value.

But it can provide an honest confidence interval.

# 20. CONSERVATIVE REPORTING

When uncertainty exists,

the system should bias toward safety.

Example

Interval:

44% – 53%

Report:

44%

or

“At least 44% remains”

Reason

False pessimism causes early ordering.

False optimism causes outages.

Outages are worse.

# 21. SELF-HEALING PROPERTY

The cold-start problem is temporary.

Eventually one of two events occurs:

- Refill

- Empty floor

Both are anchors.

Both solve steel.

Once steel is known,

all future estimates become exact.

## Important Insight

The system becomes smarter simply by existing longer.

Time creates information.

# 22. THE ROLE OF USER INPUT

User input should never be mandatory.

However,

user input can accelerate convergence.

Examples

Stamped tare weight

Cylinder type

Known refill date

These inputs are optional.

The system must remain functional without them.

# 23. V1 / V2 / V3 ESTIMATION STRATEGY

V1

Known full install

Exact estimation

V2

Partial cylinder

User-assisted estimation

V3

Autonomous estimation

Anchor-driven convergence

# 24. DOMAIN MODEL SUMMARY

The entire gas estimation problem can be summarized as:

Known:

Gross

Unknown:

Steel

Gas

Need:

Second equation

Acquire through:

Anchor Events

Solve:

Steel

Then:

Gas = Gross − Steel

Everything else in the project depends on this understanding.

END OF PART 2