# GAS CYLINDER WEIGHT MONITOR

# MASTER DESIGN DOCUMENT (MDD)

## Version 1.0

## Date: 2026-06-03

# 1. PURPOSE

This document is the canonical design reference for the Gas Cylinder Weight Monitor project.

It consolidates:

- Product vision

- LPG domain knowledge

- Measurement theory

- Calibration architecture

- ESP32-C3 architecture pivot

- UNO Q hub architecture

- Consumption analytics

- Prediction logic

- Engineering principles

- Roadmap

- Experiments

- Open questions

This document supersedes individual brainstorm notes as the primary reference.

# 2. PRODUCT VISION

## Problem

Households frequently discover an empty LPG cylinder only when cooking is about to begin.

Current solutions are reactive:

- Lift cylinder by hand

- Shake cylinder

- Guess remaining gas

- Wait for depletion

None provide reliable visibility into:

- Remaining gas

- Consumption patterns

- Run-out prediction

## Product Goal

Create a system that:

- Measures cylinder weight continuously.

- Determines remaining LPG.

- Tracks consumption patterns.

- Predicts depletion date.

- Alerts users before gas runs out.

## Core Value

The scale itself is not the product.

The product is:

Measurement → Understanding → Prediction → Action

# 3. PRODUCT EVOLUTION

## V1

Known Full Cylinder

Assumption:

User installs the system when a fresh cylinder is delivered.

Capabilities:

- Exact gas percentage

- Exact gas remaining

- Burn-rate tracking

- Days-remaining prediction

No partial-cylinder handling.

## V2

Partial Cylinder + User Assistance

User supplies:

- Stamped tare weight OR

- Brand/type information

System bootstraps percentage immediately.

## V3

Fully Autonomous

Unknown cylinder.

Unknown fill state.

No user input.

Uses:

- Interval estimation

- Anchor events

- Self-healing calibration

# 4. SYSTEM ARCHITECTURE

## Physical Architecture

Load Cell(s) ↓ HX711 ↓ ESP32-C3 ↓ WiFi UNO Q Hub ↓ Storage Analytics Prediction UI

## ESP32-C3 Responsibilities

Sensor Node

Responsibilities:

- HX711 communication

- Noise filtering

- Averaging

- Calibration

- Event detection

- Health monitoring

The ESP32 owns sensing.

## UNO Q Responsibilities

Hub

Responsibilities:

- Storage

- Analytics

- Prediction

- Reporting

- Dashboard

- Companion UI

The UNO Q owns intelligence.

# 5. THE CORE LPG MATHEMATICS

## Fundamental Equation

Gross Weight = Steel Weight + Gas Weight

G = S + g

Where:

G = gross weight S = steel weight g = gas weight

## The Central Problem

The scale measures:

G

But the user wants:

g

The system sees:

G = S + g

One equation.

Two unknowns.

Therefore:

A single measurement cannot determine gas.

This is a mathematical limitation, not a software limitation.

# 6. THE PERCENTAGE TRUTH

## Incorrect Formula

percentage = gross / full_weight

Example:

Full cylinder:

15.3 kg steel 14.2 kg gas

Total:

29.5 kg

Empty cylinder:

15.3 kg

Incorrect percentage:

15.3 / 29.5 = 52%

This means an empty cylinder appears half full.

This is the “52% Trap”.

## Correct Formula

gas_remaining = gross − steel

percentage = gas_remaining / capacity

For domestic LPG:

percentage = (gross − steel) / 14.2

An empty cylinder correctly reports:

0%

# 7. INDIAN LPG DOMAIN FACTS

## Fixed Quantities

Domestic LPG:

14.2 kg ± 150 g

Common capacities:

- 5 kg

- 14.2 kg

- 19 kg

- 47.5 kg

- 425 kg

## Variable Quantities

Steel shell weight varies.

Typical domestic range:

14.5 kg – 15.8 kg

Therefore:

Capacity should be classified.

Steel should be measured.

Never assume steel.

## Tare Weight

Every cylinder has its tare weight stamped physically.

This can be used as a manual calibration aid.

# 8. ANCHOR STRATEGY

The system learns steel weight through anchor events.

## Anchor #1: Full Install

Primary Anchor

Fresh delivery.

Known:

gas = capacity

Therefore:

steel = gross_install − capacity

This is the preferred method.

## Anchor #2: Empty Floor

Secondary Anchor

Known:

gas = 0

Therefore:

steel = gross

Useful as a cross-check.

Not guaranteed.

# 9. PARTIAL CYLINDER PROBLEM

## First Boot

Suppose:

gross = 21.6 kg

Unknown:

steel = ? gas = ?

Equation:

21.6 = steel + gas

Infinite solutions exist.

The system cannot know the exact answer.

## Mathematical Verdict

Partial cylinder at first boot is underdetermined.

No:

- AI

- ML

- Algorithm

- Sensor

can solve missing information.

## What Can Be Done

Estimate a bounded interval.

Example:

Steel range:

14.5 – 15.8 kg

Gas range:

21.6 − 15.8 = 5.8 kg

21.6 − 14.5 = 7.1 kg

Result:

Gas is between:

5.8 and 7.1 kg

Approx:

41% – 50%

## Safety Principle

Always bias toward the low-gas estimate.

Failure should be:

Order sooner.

Never:

Order too late.

# 10. WHY DELTA TRACKING ALWAYS WORKS

Day 1:

G0 = S + g0

Day 2:

G1 = S + g1

Gas used:

g0 − g1

Substituting:

(G0 − S) − (G1 − S)

Steel cancels.

Result:

Gas used = G0 − G1

Therefore:

Usage tracking works immediately.

Even when steel is unknown.

# 11. MEASUREMENT STACK

Raw HX711 Count ↓ Corrupt-Value Filtering ↓ Clean Sample ↓ Averaging ↓ Stable Reading ↓ Scale Zero Removal ↓ Gross Weight ↓ Steel Removal ↓ Gas Remaining ↓ Percentage

# 12. THE TWO TARES

## Tare #1

Scale Zero

Represents:

Empty platform baseline.

Purpose:

Remove electrical/mechanical offset.

## Tare #2

Cylinder Steel

Represents:

Empty cylinder shell.

Purpose:

Separate gas from steel.

## Rule

Never refer to both as simply:

“Tare”

Always specify:

Scale Zero

or

Cylinder Steel

# 13. QUALITY DISCIPLINE

Nothing is trusted from a single measurement.

Every quantity is validated using:

- Averaging

- Stability checks

- Independent cross-checks

# 14. NOISE CHARACTERISATION

Measure every boot:

- Standard deviation

- Peak-to-peak noise

Noise determines:

- Sample count N

- Thresholds

- Health status

N is derived.

Never arbitrary.

# 15. CALIBRATION PHILOSOPHY

## cal_factor

Converts:

Raw counts → grams

Must be:

- Multi-point

- Re-derived on ESP32-C3

- Continuously validated

## Self Validation

Use:

- Residuals

- R²

- Anchor cross-checks

- Long-term monitoring

## Continuous Trust Model

Measure → Validate → Cross-check → Trust

Not:

Measure once → Trust forever

# 16. SAMPLING STRATEGY

## Fast Internal Reads

Every few seconds.

Purpose:

- Pull requests

- Event detection

Not stored.

## Heartbeat

Every 15 minutes.

Authoritative record.

Stored permanently.

Used for:

- Analytics

- Prediction

- Leak detection

## Event Records

Generated from significant changes.

Used for:

- UX

- Session tracking

Not primary storage.

# 17. EVENT DETECTION

Events:

- Install

- Removal

- Empty Floor

- Session Start

- Session Stop

Derived from the same measurement infrastructure.

# 18. HEALTH MONITORING

Health includes:

Node Health Link Health Noise Health Calibration Health

The 15-minute heartbeat doubles as a liveness signal.

# 19. POWER STRATEGY

V1 uses:

USB Power

Reasons:

- Stationary deployment

- Immediate pull support

- Simpler firmware

- No sleep complexity

Battery operation deferred.

# 20. TRANSPORT STRATEGY

Preferred:

ESP32-C3 → WiFi → UNO Q

Reasons:

- Existing infrastructure

- Tiny data volume

- USB-powered system

- Simpler integration

BLE remains a fallback.

# 21. STORAGE PHILOSOPHY

Store everything raw.

Never discard history.

Future intelligence depends on historical data.

Initial storage:

SQLite

Simple schema.

Expandable later.

# 22. ANALYTICS

V1 Analytics:

- Used today

- Daily consumption

- Weekly consumption

- Usage patterns

Future:

- Leak detection

- Behavioral insights

- Seasonal trends

# 23. PREDICTION

Basic Model:

days_remaining = remaining_gas / burn_rate

## Burn Rate

Derived from:

Actual measurements

Never population averages.

No fixed 473 g/day assumptions.

## Prediction Evolution

Day 1: Insufficient history

Day 2+: Rough estimate

Week 1: Useful estimate

Full cycle: Highly personalized estimate

# 24. ERROR BUDGET

Minor:

±150 g fill tolerance

Major:

- cal_factor drift

- thermal drift

- mechanical creep

- wiring noise

Engineering effort should target dominant errors.

# 25. ENGINEERING PRINCIPLES

First Principles

WHY before HOW

Small → Verify → Compound

Design in Chat

Implementation via CLI

No Hardcoding

Everything derived or configurable.

Tag Everything

PROVEN DERIVED REASONED PENDING

# 26. E-000: CURRENT CRITICAL PATH

Goal:

ESP32-C3 + HX711 Bring-Up

Tasks:

- GPIO selection

- 3.3V compatibility validation

- Raw reading acquisition

- Corrupt-value filtering

- Stable measurement verification

Everything else depends on E-000.

# 27. EXPERIMENT ROADMAP

E-000 ESP32 Bring-Up

E-001 Calibration Factor Derivation

E-002 Noise Characterisation

E-003 Transport Bring-Up

005 Linearity + Hysteresis

006B Measured Consumption Validation

007B Threshold Validation

Thermal Drift

Long-Term Drift

Diurnal Drift

# 28. MVP DEFINITION

The MVP is NOT prediction.

The MVP is:

ESP32 reads weight ↓ UNO Q receives ↓ UNO Q stores ↓ UNO Q displays grams

Everything else layers on top.

# 29. LOCKED DECISIONS

LOCKED

- ESP32-C3 sensor node

- UNO Q hub

- WiFi transport

- USB power

- Hybrid sampling

- Anchor-based steel learning

- Honest percentage formula

- Single node V1

# 30. OPEN QUESTIONS

Transport details

Storage schema

Prediction refinement

Companion application

Fleet architecture

Cloud architecture

Advanced analytics

Temperature compensation

# 31. FINAL PROJECT STATEMENT

The Gas Cylinder Weight Monitor is not a weighing scale project.

It is a measurement-to-intelligence system.

The sensing layer exists to create trustworthy weight data.

The intelligence layer converts weight into gas remaining, consumption understanding, and depletion prediction.

The long-term value of the system comes not from measuring kilograms, but from helping households make better decisions before gas runs out.

END OF DOCUMENT