Gas Cylinder Monitor V1

Complete Production Revert Reference — G4 + G5 + Alert Logic

Gratian Technologies · Project 13 · Updated: 2026-06-25

Read this document before making any production changes. Every constant that differs between test and production is listed here with exact values, the file it lives in, and what it controls.

# 1. The Scaling Principle

Test mode uses scaled-down time constants so the full system behaviour can be observed within a single session using the water bowl simulator. The scaling ratio is 1:48.

Scaling ratio: 1:48

30 minutes real time = 1 day simulated

3.5 hours real time = 7 days simulated

60 minutes real time = 2 days simulated (day-alert gate)

Weight constants are NOT time-scaled.

They are scaled by physical mass: water bowl ~5kg vs real cylinder ~29kg gross.

# 2. Master Revert Table — All Constants

⚠ REVERT ALL CONSTANTS BEFORE ANY PRODUCTION TEST

Run the verification grep command in Section 4 before touching a real cylinder.

Missing even one constant causes silent wrong readings or wrong alerts.

## 2.1  Constants that MUST be reverted (12 total)

| # | Constant | File | Test value | Test meaning | Production value | Production meaning |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | NET_GAS_G | domain.py | 4535.0 | 4.535kg water | 14200.0 | 14.2kg gas (BIS fixed) |
| 2 | ANCHOR_GROSS_MIN_G | domain.py | 4800.0 | ~bowl gross | 26000.0 | min full cylinder gross |
| 3 | STEEL_PLAUSIBLE_MIN_G | domain.py | 200.0 | bowl steel ~493g | 13000.0 | min cylinder steel |
| 4 | STEEL_PLAUSIBLE_MAX_G | domain.py | 2000.0 | bowl steel upper | 18000.0 | max cylinder steel |
| 5 | REFILL_GROSS_MIN_G | domain.py | 5500.0 | above bowl gross | 29000.0 | min full cylinder gross |
| 6 | MIN_DATA_HOURS | domain.py | 0.25 (15 min) | 15 min gate | 24.0 (24 hrs) | 1 full day gate |
| 7 | BURN_RATE_WINDOW_DAYS | domain.py | 0.14583 (3.5 hrs) | rolling window | 7.0 (7 days) | 7-day rolling window |
| 8 | MIN_DAYS_FOR_DAY_ALERT | domain.py | 0.04167 (60 min) | 1hr before day-alerts active | 2.0 (2 days) | 2 days min before day-alerts |
| 9 | ALERT_AMBER_DAYS | domain.py | 0.10417 (2.5 hrs) | amber at <2.5hrs water left | 5.0 (5 days) | amber at <5 days gas left |
| 10 | ALERT_RED_DAYS | domain.py | 0.0625 (90 min) | red at <90min water left | 3.0 (3 days) | red at <3 days gas left |
| 11 | MAX_BURN_RATE_G_PER_DAY | domain.py | 100000.0 | allows test removal spikes | 2000.0 | max realistic household rate |
| 12 | HEAVY_LOAD_THRESHOLD_G | gas_monitor_v1.ino | 1000 | detects bowl weight | 2000 | detects real cylinder weight |

## 2.2  Constants that do NOT change — same in test and production

| Constant | File | Value | Why unchanged |
| --- | --- | --- | --- |
| ALERT_AMBER_G | domain.py | 2000.0 | Gram failsafe — physical threshold, not time-based |
| ALERT_RED_G | domain.py | 1000.0 | Gram failsafe — physical threshold, not time-based |
| MIN_BURN_RATE_G_PER_DAY | domain.py | 10.0 | Physical minimum — any less = sensor noise |
| REMOVAL_GRACE_S | domain.py | 120.0 | Real seconds — grace window before UNINSTALLED |
| DAILY_USE_DEFAULT_G | domain.py | 350.0 | V1 prior — shown only before burn rate ready |
| ANCHOR_SPREAD_THRESHOLD_G | domain.py | 30.0 | Physical platform noise — same hardware |
| ANCHOR_MIN_STABLE_READINGS | domain.py | 5 | 5 readings always required for anchor |
| NOISE_SIGMA_PASS_G | gas_monitor_v1.ino | 8.0 | Hardware noise floor — same sensor |
| BLE_NOTIFY_INTERVAL_MS | gas_monitor_v1.ino | 30000 | WCN3990 hardware limit — never change |

# 3. Dual-Condition Alert Logic

Alerts use two independent conditions evaluated on every reading. Either condition firing triggers the alert. This ensures correct alerts for all household types regardless of burn rate confidence.

## 3.1  Alert evaluation order

def _evaluate_alerts(gas_g, days_remaining, elapsed_days, burn_rate):

# CONDITION A: gram failsafe (always active from Day 0)

if gas_g < ALERT_RED_G: return 'RED'

if gas_g < ALERT_AMBER_G: return 'AMBER'

# CONDITION B: day-based (requires reliable burn rate)

if (days_remaining is not None and

burn_rate is not None and

elapsed_days >= MIN_DAYS_FOR_DAY_ALERT):

if days_remaining < ALERT_RED_DAYS: return 'RED'

if days_remaining < ALERT_AMBER_DAYS: return 'AMBER'

return 'NONE'

## 3.2  How alerts fire for different household types

| Household type | Burn rate | Amber fires at | Triggered by | Red fires at | Triggered by |
| --- | --- | --- | --- | --- | --- |
| Heavy cooker | 800 g/day | 4000g gas (Day ~13) | Condition B (days<5) | 2400g gas (Day ~15) | Condition B (days<3) |
| Average cooker | 350 g/day | 1750g gas (Day ~35) | Condition A (grams<2000g) | 1000g gas (Day ~38) | Condition A (grams<1000g) |
| Light cooker | 150 g/day | 750g gas (Day ~57) | Condition B (days<5) | 450g gas (Day ~60) | Condition B (days<3) |
| Day 1 guest spike | 1600 g/day (temporary) | No alert | Condition B inactive (<2 days elapsed) | No alert | Gram failsafe protects |
| Burn rate failure | None | 2000g gas | Condition A failsafe | 1000g gas | Condition A failsafe |

## 3.3  What the user sees at each alert level

| State | WebUI colour | Days display | Message |
| --- | --- | --- | --- |
| NORMAL | Green gauge | X days remaining | Gas OK |
| LOW_GAS (AMBER) | Amber gauge | X days remaining | Gas running low — consider booking |
| LOW_GAS (RED) | Red gauge | X days remaining | Gas critical — book now |
| No burn rate yet | Green gauge | — days remaining | Tracking usage... |

# 4. Burn Rate Architecture — Adaptive Lookback

Burn rate uses an adaptive strategy: cumulative from anchor when data is limited, rolling window when 7+ days of data exist. Cumulative is always the fallback.

def compute_analytics(current_gas_g):

elapsed_days = (now - anchor_ts).total_seconds() / 86400

elapsed_hours = elapsed_days * 24

# Gate: not enough data

if elapsed_hours < MIN_DATA_HOURS: return None

# Total consumed since anchor (cumulative)

total_consumed_g = NET_GAS_G - current_gas_g

if total_consumed_g <= 0: return None # noise at anchor

# Choose source

if elapsed_days < BURN_RATE_WINDOW_DAYS:

# Not enough history for rolling window

burn_rate = total_consumed_g / elapsed_days

source = 'CUMULATIVE'

else:

# Try rolling window

rolling = rolling_window_query(BURN_RATE_WINDOW_DAYS)

if rolling is not None:

burn_rate = rolling

source = 'ROLLING'

else:

# Rolling window failed (gap in cooking) — fall back

burn_rate = total_consumed_g / elapsed_days

source = 'CUMULATIVE_FALLBACK'

# Sanity bounds

if not (MIN_BURN_RATE_G_PER_DAY <= burn_rate <= MAX_BURN_RATE_G_PER_DAY):

return None

days_remaining = round(current_gas_g / burn_rate, 1)

predicted_empty = (now + timedelta(days=days_remaining)).strftime('%d %b %Y')

return {burn_rate, days_remaining, predicted_empty, source}

## 4.1  Source confidence and WebUI display

| Source | When | WebUI indicator | Confidence |
| --- | --- | --- | --- |
| CUMULATIVE | elapsed < BURN_RATE_WINDOW_DAYS | ~ prefix on days (e.g. ~3 days) | Low→Medium, improves over time |
| ROLLING | elapsed ≥ BURN_RATE_WINDOW_DAYS | No prefix (e.g. 3 days) | High — personalised to household |
| CUMULATIVE_FALLBACK | Rolling window returned None | ~ prefix on days | Medium — long-term average |
| None | elapsed < MIN_DATA_HOURS or all guards fail | — (dash) | No estimate available |

# 5. Production Verification Command

Run this before touching any real cylinder. All 11 hub constants must show production values:

grep -n 'NET_GAS_G\¦ANCHOR_GROSS_MIN_G\¦STEEL_PLAUSIBLE\¦REFILL_GROSS_MIN_G\¦MIN_DATA_HOURS\¦BURN_RATE_WINDOW\¦MIN_DAYS_FOR_DAY\¦ALERT_AMBER_DAYS\¦ALERT_RED_DAYS\¦MAX_BURN_RATE' \

hub/python/domain.py

# Expected production output:

# NET_GAS_G = 14200.0

# ANCHOR_GROSS_MIN_G = 26000.0

# STEEL_PLAUSIBLE_MIN_G = 13000.0

# STEEL_PLAUSIBLE_MAX_G = 18000.0

# REFILL_GROSS_MIN_G = 29000.0

# MIN_DATA_HOURS = 24.0

# BURN_RATE_WINDOW_DAYS = 7.0

# MIN_DAYS_FOR_DAY_ALERT = 2.0

# ALERT_AMBER_DAYS = 5.0

# ALERT_RED_DAYS = 3.0

# MAX_BURN_RATE_G_PER_DAY = 2000.0

# Also verify node firmware:

# HEAVY_LOAD_THRESHOLD_G must be 2000 in gas_monitor_v1.ino

Gas Cylinder Monitor V1 · Complete Production Revert Reference · Gratian Technologies · 2026-06-25