Gas Cylinder Monitor V1

G4 + G5 Production Readiness Reference

Gratian Technologies · Project 13 · Gas Cylinder Monitor

Purpose: read this document before making any final production changes to domain.py or any hub constants. It defines every constant, logic rule, and revert action needed to move from test/development state to production.

# 1. Design Philosophy

The system is built so that switching from development/test to production requires only changing a small set of named constants — never rewriting logic. This is the same principle used in embedded systems: one codebase, parameterised by environment.

- Test mode: uses small weights (water bowl ~5kg), short time windows (minutes/hours), low thresholds so you can observe full system behaviour in a single session.

- Production mode: uses real cylinder weights (14.2kg gas, ~29kg gross), real time windows (24h gate, 7-day rolling), production thresholds.

- The logic — formulas, state machine transitions, burn rate calculation, days remaining — is identical in both modes. Only the constants change.

- Every constant that differs between test and production is listed in this document with both values and the file/line where it lives.

⚠ CRITICAL RULE

Never revert constants one at a time. Always revert ALL constants in a group together before any production test.

Missing even one causes silent wrong readings with no error output.

# 2. G4 — Domain Logic (domain.py)

G4 owns the core state machine and all gas domain computation. It lives entirely in hub/python/domain.py. The node knows nothing about G4 — it only outputs {grams, quality, sigma}.

## 2.1  What G4 computes

| Output | Formula | When available |
| --- | --- | --- |
| gas_g | gross_g − steel_g | Immediately after anchor fires (cylinder_state = TRACKING) |
| gas_pct | gas_g / NET_GAS_G × 100 | Same as gas_g |
| alert_level | NONE / AMBER / RED | Based on gas_g vs ALERT_AMBER_G and ALERT_RED_G |
| cylinder_state | UNINSTALLED / BOOTSTRAP_ANCHOR / TRACKING / LOW_GAS | Always |
| steel_g | mean_gross − NET_GAS_G at anchor | After 5 stable anchor readings |

## 2.2  G4 state machine summary

| From state | Trigger | To state |
| --- | --- | --- |
| UNINSTALLED | User clicks Install in WebUI | BOOTSTRAP_ANCHOR |
| BOOTSTRAP_ANCHOR | 5 stable readings, spread < ANCHOR_SPREAD_THRESHOLD_G, all > ANCHOR_GROSS_MIN_G | TRACKING |
| TRACKING | gas_g < ALERT_AMBER_G | LOW_GAS |
| LOW_GAS | gas_g ≥ ALERT_AMBER_G | TRACKING (auto-clear) |
| TRACKING or LOW_GAS | gross < 500g for > REMOVAL_GRACE_S seconds | UNINSTALLED (steel cleared) |
| TRACKING or LOW_GAS | gross > REFILL_GROSS_MIN_G | BOOTSTRAP_ANCHOR (refill detected) |

## 2.3  G4 constants — REVERT REQUIRED for production

These 5 constants are currently at TEST values in domain.py. They MUST all be reverted before any production test with a real cylinder.

| Constant | Current TEST value | PRODUCTION value | Why it differs |
| --- | --- | --- | --- |
| NET_GAS_G | 4535.0 | 14200.0 | Water bowl simulation uses 4535g water. Real cylinder has 14.2kg gas (BIS IS 3196 — legally fixed). |
| ANCHOR_GROSS_MIN_G | 4800.0 | 26000.0 | Minimum gross to enter anchor collection window. Test bowl gross ~5027g. Real cylinder gross ~29kg. |
| STEEL_PLAUSIBLE_MIN_G | 200.0 | 13000.0 | Sanity check: derived steel_g must be within this range. Bowl steel ~493g. Real cylinder steel ~15kg. |
| STEEL_PLAUSIBLE_MAX_G | 2000.0 | 18000.0 | Upper bound of steel sanity check. Same reason. |
| REFILL_GROSS_MIN_G | 5500.0 | 29000.0 | Minimum gross to detect a refill in TRACKING/LOW_GAS. Must be higher than ANCHOR_GROSS_MIN_G — never conflate these two. |

## 2.4  G4 constants — same for test and production (do NOT change)

| Constant | Value | Purpose |
| --- | --- | --- |
| DAILY_USE_DEFAULT_G | 350.0 | V1 prior for days_remaining before burn rate is established |
| ALERT_AMBER_G | 2000.0 | Gas threshold for AMBER alert (~5-6 days at 350g/day) |
| ALERT_RED_G | 1000.0 | Gas threshold for RED alert (~2-3 days at 350g/day) |
| ANCHOR_SPREAD_THRESHOLD_G | 30.0 | Max spread across 5 anchor readings to accept anchor (±15g platform noise) |
| ANCHOR_STABILITY_WINDOW_G | 50.0 | Gross reading must be within this window of previous to qualify for anchor |
| ANCHOR_MIN_STABLE_READINGS | 5 | Number of consecutive stable readings required before anchor fires |
| STEEL_UNKNOWN_PRIOR_G | 16500.0 | Conservative prior when brand/steel is unknown. Underestimates gas remaining = safer failure mode. |
| REMOVAL_GRACE_S | 120.0 | Grace window before UNINSTALLED transition. Same for test and production. |

## 2.5  G4 key design principles — never violate

- gas% is always (gross_g − steel_g) / NET_GAS_G × 100. Never compute on node.

- steel_g is derived once at anchor, never hardcoded, never changed until next anchor event.

- ANCHOR_GROSS_MIN_G gates anchor collection. REFILL_GROSS_MIN_G gates refill detection. These are separate constants — REFILL must always be higher than ANCHOR.

- tare_raw absorbs the platform permanently. Rubber mats, drip trays, platform plate are mathematically invisible. Gas consumption = gross_g − steel_g. Platform fixtures cancel.

- Hub stamps all timestamps. Node has no RTC. Never trust a timestamp from the node.

- BLE notify interval is 30s (decoupled from HX711 read rate). Never set below 30s on WCN3990 hardware — crashes at 10Hz.

# 3. G5 — Analytics (domain.py)

G5 computes burn rate and days remaining from real SQLite data. It lives in domain.py alongside G4 — no separate file. The WebUI displays what G5 computes; no analytics logic ever lives in index.html.

## 3.1  What G5 computes

| Output | Formula | When available |
| --- | --- | --- |
| burn_rate_g_per_day | delta gas_g over rolling window ÷ elapsed days | After MIN_DATA_HOURS of TRACKING readings exist |
| days_remaining | gas_g ÷ burn_rate_g_per_day | After burn_rate is non-null |
| predicted_empty_date | today + days_remaining | After burn_rate is non-null |

## 3.2  The — (dash) rule — non-negotiable

CRITICAL: The system must never show a made-up number.

If MIN_DATA_HOURS of TRACKING data does not yet exist: burn_rate = None, days_remaining = None.

WebUI must display — (a dash) for these fields. Never show 0, never show a prior estimate.

This is an honesty constraint, not a display preference. Violating it destroys user trust.

## 3.3  G5 constants — REVERT REQUIRED for production

These 2 constants are time windows that are shortened for testing so you can observe G5 computing live in a single session with the water bowl. They MUST be reverted before production.

| Constant | Current TEST value | PRODUCTION value | Why it differs |
| --- | --- | --- | --- |
| MIN_DATA_HOURS | 0.5 (30 minutes) | 24.0 (24 hours) | Minimum TRACKING data before burn rate is computed. Test: 30 min lets you see it compute in one session. Production: must have at least one full day of real usage data. |
| BURN_RATE_WINDOW_DAYS | 0.25 (6 hours) | 7.0 (7 days) | Rolling window for burn rate calculation. Test: 6 hours of bowl readings. Production: 7-day rolling average accounts for weekday/weekend variation in cooking patterns. |

## 3.4  G5 burn rate calculation — how it works

The burn rate is computed by querying SQLite for TRACKING readings within the rolling window, then computing the gas_g drop over that period:

# Pseudocode — actual implementation in domain.py

window_start = now - BURN_RATE_WINDOW_DAYS * 86400

rows = db.query(

SELECT ts, gas_g FROM readings

WHERE cylinder_state = 'TRACKING'

AND ts >= window_start

ORDER BY ts ASC

)

if elapsed_hours(rows) < MIN_DATA_HOURS:

return None # — in WebUI

delta_gas_g = rows[0].gas_g - rows[-1].gas_g # first minus last = consumed

elapsed_days = (rows[-1].ts - rows[0].ts) / 86400

burn_rate_g_per_day = delta_gas_g / elapsed_days

## 3.5  G5 constants — same for test and production (do NOT change)

| Constant | Value | Purpose |
| --- | --- | --- |
| MIN_BURN_RATE_G_PER_DAY | 10.0 | Sanity floor. If computed burn rate is below this, treat as sensor noise and return None. |
| MAX_BURN_RATE_G_PER_DAY | 2000.0 | Sanity ceiling. If above this, something is wrong (disturbance, removal). Return None. |

## 3.6  G5 key design principles — never violate

- Only TRACKING readings count toward burn rate. BOOTSTRAP_ANCHOR and UNINSTALLED readings are excluded — those are not real consumption.

- burn_rate_g_per_day is always derived from real SQLite data. No population averages, no government statistics, no hardcoded prior.

- days_remaining = gas_g / burn_rate_g_per_day. Simple division. No smoothing, no weighting. Accuracy comes from the rolling window, not from a clever formula.

- If burn_rate is None, days_remaining is None. No fallback to DAILY_USE_DEFAULT_G in the WebUI output — that constant is only used internally as a placeholder before G5 is ready.

- G5 lives in domain.py. The WebUI only renders what domain.py returns. No analytics logic in index.html.

# 4. Master Production Revert Checklist

Before any production test with a real 14.2kg cylinder, ALL of the following must be reverted. Check each one individually.

⚠ DO THIS BEFORE TOUCHING A REAL CYLINDER

Run: grep -n 'NET_GAS_G\¦ANCHOR_GROSS_MIN_G\¦STEEL_PLAUSIBLE\¦REFILL_GROSS_MIN_G\¦MIN_DATA_HOURS\¦BURN_RATE_WINDOW' hub/python/domain.py

Verify every value matches the PRODUCTION column below before proceeding.

| # | Constant | File | Test value | Production value | ✔ |
| --- | --- | --- | --- | --- | --- |
| 1 | NET_GAS_G | domain.py | 4535.0 | 14200.0 |  |
| 2 | ANCHOR_GROSS_MIN_G | domain.py | 4800.0 | 26000.0 |  |
| 3 | STEEL_PLAUSIBLE_MIN_G | domain.py | 200.0 | 13000.0 |  |
| 4 | STEEL_PLAUSIBLE_MAX_G | domain.py | 2000.0 | 18000.0 |  |
| 5 | REFILL_GROSS_MIN_G | domain.py | 5500.0 | 29000.0 |  |
| 6 | MIN_DATA_HOURS | domain.py | 0.5 | 24.0 |  |
| 7 | BURN_RATE_WINDOW_DAYS | domain.py | 0.25 | 7.0 |  |
| 8 | HEAVY_LOAD_THRESHOLD_G | gas_monitor_v1.ino | 1000 | 2000 |  |

Note: HEAVY_LOAD_THRESHOLD_G (item 8) is a node-side constant in the ESP32-C3 firmware. It requires a reflash via Arduino IDE (COM11, ESP32C3 Dev Module, esp32 by Espressif v3.0.7). All other constants are hub-side in domain.py and take effect after docker redeploy.

# 5. Production Deploy Procedure

## 5.1  Hub constants (domain.py)

After reverting all 7 hub constants in domain.py:

cd ~/ArduinoApps/gas-cylinder-monitor/hub

bash deploy.sh

# Verify constants took effect:

docker exec gas-cylinder-monitor-hub-main-1 \

python3 -c "import domain; print(domain.NET_GAS_G, domain.ANCHOR_GROSS_MIN_G, \

domain.REFILL_GROSS_MIN_G, domain.MIN_DATA_HOURS, domain.BURN_RATE_WINDOW_DAYS)"

# Expected output:

# 14200.0 26000.0 29000.0 24.0 7.0

## 5.2  Node firmware (HEAVY_LOAD_THRESHOLD_G)

- Open Arduino IDE on Windows

- Open node/gas_monitor_v1/gas_monitor_v1.ino

- Locate HEAVY_LOAD_THRESHOLD_G and change from 1000 to 2000

- Board: ESP32C3 Dev Module | Port: COM11 | Package: esp32 by Espressif v3.0.7

- Upload. If upload fails, lower speed to 115200. Never use Erase All Flash.

- Verify in Serial Monitor: N-TARE-CHECK line should show threshold=2000

## 5.3  Post-revert verification

- Hub: docker logs show correct gas% on first reading after bowl/cylinder placed

- Node: boot log shows N-TARE-CHECK with correct 2000g threshold

- WebUI: Install cylinder button visible when state=UNINSTALLED

- WebUI: After install, gas% shows ~100%, days_remaining shows — until MIN_DATA_HOURS passes

- WebUI: After MIN_DATA_HOURS, burn_rate and days_remaining appear

# 6. Quick Reference — What Goes Where

| Concern | Owns it | Never owns it |
| --- | --- | --- |
| gas%, gas_g, steel_g | domain.py (G4) | node firmware, index.html |
| burn_rate, days_remaining | domain.py (G5) | node firmware, index.html |
| cylinder_state, alert_level | domain.py (G4) | node firmware, index.html |
| Weight in grams | node firmware (HX711) | hub, WebUI |
| Timestamps | hub (on receipt) | node (no RTC) |
| All display/rendering | index.html (WebUI) | domain.py, node |
| SQLite reads (for G5) | domain.py | index.html, node |
| All named constants | domain.py (hub) or .ino (node) | never hardcoded inline |

Gas Cylinder Monitor V1 · Gratian Technologies · Project 13 · Produced: 2026-06-25