Load Cell  →  HX711  →  MCU

A First-Principles Deep Dive

Original Learning Notes  |  Corrections  |  Additions

Learning Philosophy

Not what, but why and how it actually works underneath.

Small structured stages — no overwhelming jumps.

Hands-on always — every concept tied to real code.

System-level clarity — every new concept links back to what you already know.

# The System You Are Observing

This document covers the first three layers of a weight measurement pipeline used in IoT / embedded systems. Understanding these layers from the physics up is essential before writing a single line of code.

| PIPELINE | Weight → Load Cell → HX711 → MCU → (Bridge → Python → UI) |
| --- | --- |

We focus exclusively on the first three layers:

- Load Cell — physics and analog signal generation

- HX711 — analog-to-digital conversion

- MCU — bit-level digital reading

# Case 1: Static Load — Container Placed on Load Cell

## Layer 1: The Load Cell (Physics + Electrical)

### What Physically Happens

A load cell is a precision metal beam. When weight is applied:

- It bends slightly — on the order of micrometers

- This mechanical deformation is called strain

### Inside: The Wheatstone Bridge

The load cell contains four strain gauges wired as a Wheatstone bridge. Strain changes their electrical resistance:

| No Load | With Load |
| --- | --- |
| R1 = R2 = R3 = R4 (balanced) | R1 ↑ R2 ↓ R3 ↑ R4 ↓ (unbalanced) |
| Bridge output = 0 mV | Bridge output = few mV |

The voltage produced is extremely small — typically 1–10 mV depending on the weight and load cell rating.

## Layer 2: The HX711 (Analog → Digital Conversion)

### What HX711 Receives

The HX711 receives a differential voltage across its A+ and A− pins. This is the millivolt-range signal from the load cell bridge.

### Step 1: Amplification via PGA

The HX711 contains a Programmable Gain Amplifier (PGA). Default gain is 128:

3 mV × 128 = 384 mV   (internal amplified signal)

This amplification is necessary because the ADC needs a larger input range to produce an accurate result.

### Step 2: Sigma-Delta ADC

HX711 uses a Sigma-Delta (Σ-Δ) ADC. Rather than taking one instantaneous measurement, it:

- Oversamples the input thousands of times within the conversion window

- Integrates accumulated error through a feedback loop

- Produces a single high-resolution 24-bit output

### Step 3: The Output

The result is a 24-bit signed integer. Example:

0000 0001 1010 1010 1100 0101

This number is proportional to strain, which is proportional to weight. It is NOT a weight value by itself.

### Step 4: Data Ready Signal

When conversion is complete, HX711 drives its DOUT pin LOW. This signals the MCU: data is ready, come read it.

## Layer 3: MCU — Bit-Level Reading

### Detecting Data Ready

if (DOUT == LOW) {

// data is ready

read_24_bits();

}

### Reading Each Bit (Serial Clock Protocol)

For each of the 24 bits, the MCU does the following:

- Drive SCK HIGH

- Wait

- Drive SCK LOW

- Read DOUT at the falling edge — the bit is valid here

Code equivalent:

for (int i = 0; i < 24; i++) {

SCK = HIGH;

wait();

SCK = LOW;

bit = digitalRead(DOUT);

value = (value << 1) | bit;

}

### Gain Selection Pulses

After reading 24 bits, the MCU sends extra clock pulses to set the gain for the NEXT conversion:

25 pulses total  →  next gain = 128 (channel A)

26 pulses total  →  next gain =  32 (channel B)

27 pulses total  →  next gain =  64 (channel A)

| KEY | These gain pulses are part of the same read transaction — not a separate configuration step. This is elegant hardware design: no registers needed. |
| --- | --- |

### Final Raw Value

raw_value = 8,324,512  (example)

This is NOT weight. It is proportional to strain, which is proportional to weight. Conversion requires calibration.

# Case 2: Continuous Change — Water Slowly Decreasing

## Layer 1: Load Cell Over Time

As water is consumed, mass decreases gradually. The load cell beam slowly returns toward its resting shape:

R1 slowly ↓    R2 slowly ↑

R3 slowly ↓    R4 slowly ↑

Vout decreases gradually (continuous analog)

This change is continuous — it is never step-based.

## Layer 2: HX711 Continuous Sampling

HX711 operates at 10 Hz (default) or 80 Hz (configured). At 10 Hz, one new conversion is produced every 100 ms.

Output over time looks like:

t0 →  8,324,512

t1 →  8,324,400

t2 →  8,324,200

t3 →  8,323,800

...

This is a smooth, gradually decreasing time series — a discrete representation of the continuous physical reality.

## Layer 3: MCU Reads the Stream

The MCU runs a polling loop:

if (is_ready()) {

value = read();

}

Over time, the MCU accumulates a time series. Even when the weight is stable, the readings will not be perfectly identical due to noise.

# Noise: Critical Understanding

## What Noise Looks Like

Even with a perfectly stable, unmoving load:

8,324,512

8,324,505

8,324,518

8,324,490

## Sources of Noise

| Source | Example 1 | Example 2 |
| --- | --- | --- |
| Mechanical | Table vibration | Uneven load placement |
| Electrical | Power supply ripple | EMI from nearby devices |
| ADC | Quantization error | Internal thermal noise |

## Solving Noise: Averaging + Stability Detection

### Step 1: Collect N Samples

samples = [8324512, 8324505, 8324518, 8324490, 8324500]

### Step 2: Compute Average

avg = sum(samples) / len(samples)  =  8,324,505

### Step 3: Check Stability

spread = max(samples) - min(samples)

= 8,324,518 - 8,324,490

= 28

if spread < threshold (e.g. 100):

reading is STABLE

### Step 4: Output Decision

if stable:

output avg

else:

discard or hold previous value

| PRACTICE | Sample count: 5–10 per stability check Threshold: ~50–200 raw units (calibrate to your environment) |
| --- | --- |

# The Math: mV → 24-bit → Human-Readable Number

## Part A: Why the Range Is ±8,388,607?

### A 24-bit Signed Integer (2's Complement)

A signed 24-bit number has 2^24 = 16,777,216 total values. Half are negative, half positive:

Minimum = -2^23       = -8,388,608

Maximum =  2^23 - 1   =  8,388,607

These are mathematical limits of the bit width. They are not HX711-specific.

### Mapping Voltage to Integer (Linear Mapping)

HX711 maps the input voltage range linearly to this integer range:

digital_value = (Vin / Vfull_scale) × (2^23 - 1)

Where:

Vin          = input voltage from load cell

Vfull_scale  = 20 mV  (at gain 128, AVDD = 5V)

2^23 - 1     = 8,388,607

### Example Calculation

Input: Vin = 3 mV

digital_value = (3 / 20) × 8,388,607

= 0.15 × 8,388,607

≈ 1,258,291

### Converting to 24-bit Binary

1,258,291  in 24-bit binary:

0001 0011 0010 0100 1111 0011

## Part B: How MCU Reconstructs Number from Bits

### The Operation: value = (value << 1) | bit

This is serial-to-parallel conversion. Let's trace through a 4-bit example with input bits: 1 0 1 1

| Step | Binary operation | Decimal value |
| --- | --- | --- |
| Start | value = 0000 | 0 |
| Bit 1 = 1 | 0000 << 1 = 0000, then ¦ 1 = 0001 | 1 |
| Bit 2 = 0 | 0001 << 1 = 0010, then ¦ 0 = 0010 | 2 |
| Bit 3 = 1 | 0010 << 1 = 0100, then ¦ 1 = 0101 | 5 |
| Bit 4 = 1 | 0101 << 1 = 1010, then ¦ 1 = 1011 | 11 |

Result: binary 1011 = decimal 11. The MCU has reconstructed the number from individual bits.

| INSIGHT | Each new bit: shift existing number left (make room), then insert the new bit at the rightmost position. This is how all serial digital protocols work. |
| --- | --- |

# Timing: What Happens in the 100 ms Window?

## Load Cell (Continuous Analog)

The load cell voltage changes continuously. There is no concept of ‘updates’ — it is always reflecting the current physical state:

t = 0 ms   →  3.000 mV

t = 20 ms  →  2.995 mV

t = 50 ms  →  2.990 mV

t = 100 ms →  2.980 mV

## HX711 (Internal Averaging Window)

The HX711 does NOT sample once at the 100 ms mark. It samples continuously throughout the entire 100 ms window and integrates:

- Tens of thousands of internal measurements taken across 100 ms

- Sigma-Delta modulator produces a 1-bit bitstream whose density encodes voltage

- Decimation filter converts that bitstream into one 24-bit output number

The result is effectively a time-averaged value over the entire 100 ms window — which is why HX711 is naturally noise-resistant.

## The Full Timing Sequence

t = 0 ms      :  Conversion starts

t = 0–100 ms  :  HX711 samples continuously (internal)

t = 100 ms    :  Conversion complete — DOUT = LOW

:  [WAITING STATE — no new sampling]

t = 100+ ms   :  MCU detects DOUT LOW, reads 24 bits

:  MCU sends gain pulses (25/26/27)

:  DOUT goes HIGH

t = next       :  New conversion begins immediately

## What If MCU Does Not Read?

If the MCU delays or never reads:

- HX711 holds DOUT LOW and freezes its output

- No new conversion starts

- The load cell voltage keeps changing physically, but HX711 is no longer sampling it

- Those physical changes are effectively lost

| DESIGN IMPLICATION | HX711 is MCU-controlled, not free-running. The MCU is the gatekeeper that triggers each new conversion by completing the read of the previous one. |
| --- | --- |

# Calibration: Raw Value → Grams → Kilograms

## The Formula

weight_kg = (raw - offset) / cal_factor

Where:

raw        = current raw reading from HX711

offset     = raw reading at zero weight (tare)

cal_factor = raw units per kilogram

## Step-by-Step Example

### Step 1: Establish Tare (Offset)

With nothing on the scale, read the raw value:

offset = 58,000   (example zero-load reading)

### Step 2: Read With Known Weight

Place a known mass (e.g. 10 kg) on the scale:

raw_with_weight = 1,258,000

### Step 3: Compute Calibration Factor

cal_factor = (raw_with_weight - offset) / actual_weight

= (1,258,000 - 58,000) / 10

= 1,200,000 / 10

= 120,000  raw units per kg

### Step 4: Convert Any Future Reading

raw = 1,258,000

net = 1,258,000 - 58,000  =  1,200,000

weight = 1,200,000 / 120,000  =  10.0 kg

# Corrections to the Original Notes

## Correction 1: The ±20 mV Range Is Not Fixed

| CORRECTION | The original notes treat ±20 mV as a constant specification. Reality: The full-scale input range is derived from supply voltage and gain: Full-scale = ±0.5 × AVDD / Gain At AVDD = 5V, Gain = 128: ±0.5 × 5.0 / 128 = ±19.5 mV ≈ ±20 mV If your AVDD is 3.3V instead: ±0.5 × 3.3 / 128 = ±12.9 mV This affects your calibration accuracy. Always verify AVDD in your circuit. |
| --- | --- |

## Correction 2: ‘HX711 Goes Blind’ Timing is Slightly Imprecise

| CORRECTION | The original notes say HX711 is 'effectively blind' while waiting for the MCU. More precisely: the blindness occurs AFTER conversion is complete, during the waiting state (DOUT LOW, MCU hasn't read yet). The 100 ms integration window is always a FULL active sampling period. HX711 never freezes mid-conversion. Corrected timeline: [0–100 ms] : HX711 actively integrating (NOT blind) [100 ms+] : Conversion done, DOUT LOW, waiting (BLIND to new changes) [After read] : New conversion begins immediately (active again) |
| --- | --- |

## Correction 3: Sigma-Delta Is More Than Simple Averaging

| CORRECTION | The original notes say HX711 'averages thousands of internal samples.' This is a useful intuition but not mechanistically accurate. What actually happens: 1. The sigma-delta MODULATOR runs continuously, producing a 1-bit bitstream 2. The density of 1s in that bitstream encodes the input voltage 3. A DECIMATION FILTER converts the bitstream into one 24-bit number Why this matters: - The output is NOISE-SHAPED, not just averaged - High-frequency noise is pushed out of the measurement band - This is why HX711 produces clean readings even with noisy power supplies - Simple averaging would not achieve this quality |
| --- | --- |

## Correction 4: Gain Pulses Occur During the Read, Not After

| CORRECTION | The original notes describe gain pulses as happening 'after 24 bits'. More precisely: pulses 25, 26, and 27 are part of the SAME read transaction. They are not a separate configuration step that follows the read. Total pulse count in one transaction determines next gain: 25 pulses → next gain = 128 (channel A) 26 pulses → next gain = 32 (channel B) 27 pulses → next gain = 64 (channel A) This is intentional hardware design: no registers, no configuration bus. |
| --- | --- |

# Additions: What Is Also Worth Knowing

## Addition 1: Two-Point Calibration Is Better Than One-Point

| ADDITION | The original notes cover one-point calibration (tare + one known weight). In real deployments, two-point calibration is significantly more accurate. One-point: Assumes your tare offset is perfectly stable. Two-point: Measures at TWO known weights, derives both offset and slope. Why this matters for gas cylinder monitoring: - Temperature drift slowly shifts the offset over hours and days - A one-point calibration done in the morning will drift by afternoon - Two-point calibration captures both offset and scale factor correctly - Periodic re-tare (when cylinder is known empty) is good practice |
| --- | --- |

## Addition 2: The Nyquist Consideration

| ADDITION | HX711 at 10 Hz means it can accurately track changes up to 5 Hz (Nyquist theorem). For a gas cylinder losing weight slowly over hours, this is more than sufficient. But if you ever need to track rapid weight events (e.g. a refill), switch to 80 Hz mode so rapid changes are not missed. Nyquist rule: sampling rate must be at least 2× the fastest change you want to detect. |
| --- | --- |

## Addition 3: Why the HX711 Protocol Is Smart Design

| ADDITION | The HX711 communicates entirely through two GPIO pins: SCK and DOUT. No UART, no SPI, no I2C bus — just clock and data. Why this is clever: - MCU controls conversion rate by controlling when it reads - Gain is set via pulse count — no register writes needed - DOUT LOW doubles as both 'data ready' signal and data output - Extremely simple to implement on any MCU, even tiny ones Trade-off: MCU must poll DOUT. If MCU is busy, conversions stall. |
| --- | --- |

## Addition 4: Calibration Drift in Long-Running Systems

| ADDITION | For a system monitoring gas cylinders over months: - Load cell creep: prolonged load causes slow permanent deformation - Temperature coefficient: strain gauge resistance changes with temperature - ADC reference drift: internal voltage reference shifts slightly over time Practical mitigations: - Log raw values alongside calibrated values for diagnostics - Implement a software re-calibration trigger (e.g. when cylinder known empty) - Use temperature compensation if your environment has wide swings |
| --- | --- |

# The Complete Mental Model (Lock This)

| LOAD CELL | Continuous analog physics. Always reflecting current weight. Never discrete. |
| --- | --- |
| HX711 ADC | Integrates continuously for 100 ms window. Produces one 24-bit signed integer. Noise-shaped by sigma-delta architecture. |
| DOUT SIGNAL | Goes LOW when conversion complete. MCU must pull it. HX711 does not push. |
| MCU | Reads 24 bits serially via clock pulses. Reconstructs number bit by bit. Controls gain via pulse count. |
| NOISE | Always present. Mitigated by averaging N samples and checking spread against a threshold. |
| CALIBRATION | Raw value is proportional to weight. Convert via: weight = (raw - offset) / cal_factor |
| TIMING | MCU unlocks next conversion by completing the read. HX711 is MCU-paced, not free-running. |

Weight → Strain → Resistance → Voltage → Amplified → Digitized → Bits → Number → Calibrated → kg