Gas Cylinder Monitor

Session Decisions & Architecture Reference

Chat session: 2026-06-16

Gratian Technologies / Project13

Platform: ESP32-C3 node + Arduino UNO Q AQ3 hub

This document captures every design decision, first-principles derivation, architecture choice, and backlog item discussed in the 2026-06-16 session. It is the authoritative single reference — read this before asking any question that was covered in this session.

# 1. The Cold-Start Problem — First Principles

The fundamental constraint of the system: the HX711 outputs raw ADC counts. To convert counts to grams, a cal_factor is needed. To compute gas%, steel tare must be known. On day 1 with no history, neither is available from sensor data alone.

The core equation:

G (gross) = S (steel) + g (gas)

gas% = (G - S) / 14200 * 100

One equation. Two unknowns (S and g). Cannot be solved from a single reading. This is not a software problem — it is physics. Any system that claims to solve it without at least one external reference is hiding the reference somewhere.

Where references come from — three legitimate sources:

- BIS IS 3196 law: net gas = exactly 14.2 kg for every domestic cylinder in India. This is the free reference for fresh cylinders.

- Stamped tare on cylinder collar: steel weight physically stamped under Legal Metrology rules. One-time user read.

- Refill anchor event: hub observes raw_full and raw_empty across a complete cycle. cal_factor = (raw_full - raw_empty) / 14200. Available after first refill only.

# 2. Three Product Versions — Cold-Start Strategy

All three versions handle the one-time first-install scenario differently. After the first fresh cylinder is delivered and the anchor event fires, all three versions are permanently identical — V1 logic runs forever.

| Version | Install Condition | How Steel is Known | User Input | Accuracy | anchor_exact |
| --- | --- | --- | --- | --- | --- |
| V1 | Fresh full cylinder delivered at install | S = G_install - 14.2 (BIS law). Exact. | None ever | Exact from first reading | true |
| V2 | Partial cylinder, brand known | User selects brand (HP/Bharat/Indane) OR types stamped tare once | Once at first boot only | ~±1-3% until first anchor | false → true |
| V3 | Partial cylinder, fully blind | Interval: gas ∈ [G-15.8, G-14.5]. Midpoint S=15.15kg | None ever | ±5% interval until anchor | false → true |

### V2 Methods — Two Options

- Method A — Stamped tare: user reads kg stamped on cylinder collar, types into dashboard once. S = stamped_value. Exact.

- Method B — Brand selection: user presses HP / Bharat / Indane button once. System uses brand average steel from lookup table.

| Brand | Avg Steel (kg) | Gas% Error |
| --- | --- | --- |
| Indane (IOC) | 14.8 | ±1-3% |
| HP (HPCL) | 15.7 | ±1-3% |
| Bharat (BPCL) | 15.3 | ±1-3% |

### V3 Interval Formula

gas_min = gross - 15.8 kg   (steel_max)

gas_max = gross - 14.5 kg   (steel_min)

Both clamped to [0, 14200g]

Interval width = ~650g = ~5%

The interval position IS the classification. No branching logic needed. Near top = nearly full. Near middle = about half. Near zero = nearly empty. Always report conservative (low-gas) end for warnings — never tell user they have more gas than they do.

Delta tracking works from day 1 in ALL versions:

used = weight_earlier - weight_now

Steel cancels in subtraction. Burn rate, sessions, days_remaining all exact immediately.

Only the absolute gas% gauge is fuzzy in V3 until the anchor event.

# 3. Special Cases the Hub Must Handle

| Scenario | Hub Behaviour |
| --- | --- |
| Partial same brand at install | User selects brand → V2 Method B. Steel from lookup. anchor_exact: false. Heals at next refill. |
| Partial different/unknown brand | User cannot identify brand → falls to V3 interval mode. Heals at next refill. |
| Non-domestic cylinder | Derived S outside 14.5-15.8 kg range → FLAG alert. Do NOT write poisoned steel to config.json. Do not compute gas%. |
| Old cylinder not fully empty when new one arrives | G-jump AND G>26kg both required. Partial-swap cannot trigger anchor. Safe. |
| Cylinder already on platform at first boot (V1) | System ignores it. Stays UNINSTALLED. Waits for next fresh delivery G-jump. |

# 4. Threshold Derivations — First Principles

All three thresholds are derived from the physical weight space of Indian domestic LPG cylinders. No magic numbers. No assumptions.

## 4.1 Physical Weight Space

| Object | Weight Range | Notes |
| --- | --- | --- |
| Empty platform | ~0 kg | Noise, dust, residual only |
| Kitchen objects (pots, bags) | 0.5 - 5 kg | Largest credible false event |
| GAP | 5 - 14.5 kg | Nothing real lives here |
| Empty cylinder (steel only) | 14.5 - 15.8 kg | Indane min to HP max |
| Partial cylinders | 15.8 - 29 kg | Various gas levels |
| Full domestic cylinder | 29.0 - 30.0 kg | Steel + 14.2 kg gas |

## 4.2 CYLINDER_REMOVED_KG = 2 kg

Separates: empty platform (max ~1 kg) from lightest real cylinder (14.5 kg steel). Gap = 13.5 kg. Pick 2 kg — sits comfortably low in the gap. Rejects false empty declarations. Accepts no real cylinder as empty.

## 4.3 REFILL_THRESHOLD_KG = 6 kg

Separates: largest false jump (5 kg pot) from real cylinder jump (~29 kg). Gap = 24 kg. Pick 6 kg — just above noise ceiling. First-principles rule: set threshold at minimum safe distance above noise ceiling, not arbitrarily high. 10 kg or 12 kg also work, but buy nothing extra while reducing sensitivity margin unnecessarily.

## 4.4 FRESH_CYLINDER_MIN_KG = 26 kg

Guards against partial-cylinder swap false trigger. Lightest full domestic cylinder: Indane 14.8 + 14.2 = 29.0 kg. Heaviest partial: HP 15.7 + ~12 kg gas = ~27.7 kg. Gap = 1.3 kg. Pick 26 kg — 3 kg below lightest full cylinder (measurement headroom), above heaviest partial.

## 4.5 Why Both Conditions Are Required (AND gate)

Jump > 6 kg alone: passes on partial swap (e.g. 20 kg removed, 27 kg placed — jump = 7 kg).

G > 26 kg alone: passes on cylinder already on platform at boot (no jump event, but weight present).

Both together: unambiguously identifies only one real-world event — a fresh full domestic cylinder placed on an empty platform.

## 4.6 Stability Gate — 3 Consecutive Windows

Both conditions must hold for 3 consecutive measurement windows before anchor fires. Prevents spurious triggers from mechanical bounce, vibration, or transient readings during cylinder placement.

# 5. V1 Hub State Machine — Complete Design

| State | Entry Condition | Outputs | Exit Conditions |
| --- | --- | --- | --- |
| UNINSTALLED | No cylinder block in config.json | Dashboard: Place new cylinder. No gas outputs. | G-jump > 6 kg AND G_new > 26 kg AND stable 3 windows → TRACKING |
| TRACKING | cylinder block present in config.json | gas_kg, gas_pct, burn_rate (after 24h), days_remaining (after 24h) | G < 2 kg → UNINSTALLED ¦ gas < 2 kg OR days < 10 → LOW GAS |
| LOW GAS | gas_kg < LOW_GAS_KG or days < LOW_GAS_DAYS | Same as TRACKING + BLE alert + WebUI warning | G-jump > 6 kg (refill) → TRACKING ¦ G < 2 kg → UNINSTALLED |

## 5.1 Anchor Event — What Fires When Fresh Cylinder Detected

- G-jump > 6 kg AND G_new > 26 kg both true for 3 consecutive windows

- S = G_new - 14.2 kg computed

- Validate S in range [14.5, 15.8] kg. Outside range = non-domestic → FLAG, abort

- S written to config.json cylinder block. anchor_exact: true.

- State transitions UNINSTALLED → TRACKING

- gas_kg and gas_pct go live immediately

## 5.2 Cylinder Change Calibration Sequence (4 Steps, Locked Order)

Fires on every cylinder removal + replacement. Order is critical — cal_factor must be validated BEFORE G_new is measured.

| Step | Trigger | Action | Why This Order |
| --- | --- | --- | --- |
| Step 1 | G < 2 kg | Capture raw_empty. Lock empty platform reading. | Must capture before any re-zeroing |
| Step 2 | After Step 1 | Auto-tare. Update tare_raw to empty platform. Corrects zero-drift silently. | Must tare before validating cal_factor |
| Step 3 | After Step 2 (skip on first cylinder ever) | Validate cal_factor vs previous cycle data. If drift > 3%, update and log. | Must validate before measuring new cylinder |
| Step 4 | New cylinder placed | Measure G_new with fresh tare + validated cal_factor. Derive new S. | Measurement must use corrected references |

# 6. Locked Constants — Never Change

| Constant | Value | Source / Justification |
| --- | --- | --- |
| CAPACITY_KG | 14.2 kg | BIS IS 3196 — legally mandated net gas weight for Indian domestic cylinders |
| REFILL_THRESHOLD_KG | 6.0 kg | Derived: above max kitchen noise (5 kg), well below min real jump (29 kg) |
| FRESH_CYLINDER_MIN_KG | 26.0 kg | Derived: below lightest full cylinder (29 kg), above heaviest partial (27.7 kg) |
| CYLINDER_REMOVED_KG | 2.0 kg | Derived: above empty platform noise (<1 kg), below lightest cylinder (14.5 kg) |
| LOW_GAS_KG | 2.0 kg | Alert fires when gas remaining < 2 kg |
| LOW_GAS_DAYS | 10 days | Alert fires when days remaining < 10 |
| STEEL_MIN_KG | 14.5 kg | Minimum plausible domestic steel (Indane) |
| STEEL_MAX_KG | 15.8 kg | Maximum plausible domestic steel (HP) |
| MIN_HOURS_BURNRATE | 24 hours | Burn rate not shown until 24h of real data exists. Never assume. |
| BURN_RATE_WINDOW_DAYS | 7 days | Rolling 7-day average for burn rate after initial 24h gate |
| 473 g/day | NEVER USED | Government average. Banned from system. Every number from this household only. |

# 7. Complete Backlog — What Remains to Build

Organised by layer. Build order is top to bottom — nothing in a layer can be built until the layer below it is verified.

## Layer 1 — Node Foundation (ESP32-C3)

| Item | Status | Description |
| --- | --- | --- |
| 1A — Modular sketch port | PARTIAL | 004 modular done on STM32. Must port hx711/tare/noise/cal/weight as .h/.cpp to ESP32-C3 3-cell. Verify identical output. |
| 1B — Load cell health detection | NOT STARTED | Detect per-cell failures: open wire (zero output), stuck reading, erratic variance. Flag in quality field. Not designed. |
| 1C — Timing instrumentation | NOT STARTED | Time each boot phase with millis(): settle_ms, tare_ms, noise_char_ms, cal_ms. Emit in journal. No RTC — sequence-stamped durations only. |
| 1D — Structured Serial journal | NOT STARTED | Format: [BOOT] phase=TARE result=SUCCESS spread=142 ms=2341. Every phase, every quality decision, every error. Not designed. |

## Layer 2 — Experiments (all must run on 3-cell ESP32-C3 — single-cell results void)

| Experiment | Status | What It Answers |
| --- | --- | --- |
| 3E-001 cal_factor | DONE | ~36 raw/g. 3-cell parallel. Locked. |
| 3E-002 noise floor | DONE | STD 4.64g BLE-on. Threshold 18.54g. Locked. |
| E-005 linearity | DONE | Linear 200-1700g. One cal_factor valid across range. Locked. |
| 3E-004 cal+run | DONE | Self-calibrating boot. ±7g accuracy verified. Production sketch. |
| 3E-006B min detectable removal | NOT RUN | What is the smallest weight removal the system reliably detects? Needs water container + measuring cup. |
| 3E-007B false positive rate | NOT RUN | How many false triggers over 30 min stable load? Validates threshold_g derivation. |
| 3E-008 temperature drift | NOT RUN | How much does cal_factor and zero shift with kitchen temperature variation? |
| 3E-009 long-run 6hr stability | NOT RUN | Baseline wander + mechanical creep under real load for 6 hours. |
| 3E-010 load cell failure injection | NOT DESIGNED | Disconnect one cell deliberately. Does health module detect and flag it correctly? |
| 3E-005 V1 anchor validation | NOT RUN | Simulate fresh cylinder with water container. Hub detects G-jump, derives steel, gas% live. End-to-end gate for Group 4. |

## Layer 3 — Hub (AQ3 Python)

| Item | Status | Description |
| --- | --- | --- |
| BLE subscriber | RUNNING | Receives {grams, quality, sigma} from ESP32-C3. Deployed. |
| SQLite + WebUI skeleton | RUNNING | Deployed at AQ3:7000. No gas outputs yet. |
| Hub timestamp discipline | UNVERIFIED | Hub must stamp ALL receipts with Python datetime on receipt. ESP32 has no RTC. Verify working end-to-end. |
| Hub structured event journal | NOT BUILT | Log every anchor event, state transition, quality=FAILED receipt, cal_factor update. Searchable audit trail. |
| quality=FAILED handling | NOT DESIGNED | What does hub do when node reports FAILED? Skip reading? Alert? Defined behaviour needed. |
| Group 4 — Domain logic | NOT BUILT | Steel derivation + state machine UNINSTALLED/TRACKING/LOW GAS. Gate: 3E-005 must pass first. |
| Group 5 — Analytics | NOT BUILT | Daily consumption from heartbeat spine. Rolling 7-day burn rate. 24h gate enforced. |
| Group 6 — Prediction | NOT BUILT | days_remaining = gas_remaining / burn_rate. Shows — until 24h gate. Confidence interval. |
| Group 7 — WebUI outputs | NOT BUILT | Gas gauge (% + kg), state display, days remaining, trend chart. Live via Socket.IO. |
| V2 cold-start logic | NOT BUILT | Brand lookup table + stamped tare input flow. anchor_exact: false path. Heals at anchor. |
| V3 cold-start logic | NOT BUILT | Interval estimation formula. Conservative bias. Delta tracking exact from day 1. |

# 8. Correct Build Order — Nothing Skipped

This is the sequence. Each step must be verified before the next starts. No jumping ahead.

- 1A — Modular sketch port to 3-cell ESP32-C3

- 1B — Load cell health detection module

- 1C — Timing instrumentation (millis() per phase)

- 1D — Structured Serial journal on node

- 3E-006B — Min detectable removal experiment

- 3E-007B — False positive rate experiment

- 3E-008 — Temperature drift experiment

- 3E-009 — Long-run 6hr stability experiment

- 3E-010 — Load cell failure injection experiment

- Hub timestamp discipline verification

- Hub structured event journal

- Hub quality=FAILED handling design + implementation

- Group 4 domain logic (steel derivation + state machine)

- 3E-005 — V1 anchor validation experiment (water container simulation)

- Group 5 — Analytics (burn rate from real data)

- Group 6 — Prediction (days remaining)

- Group 7 — WebUI outputs (gas gauge, trend, state)

- V2 cold-start logic (brand lookup + stamped tare path)

- V3 cold-start logic (interval estimation)

# 9. Water Container Simulation — Experiment Design

For 3E-005 and 3E-006B, we simulate a fresh cylinder using a water container. The hub sees only grams — it cannot tell the difference between a cylinder and a container of the same weight.

## 9.1 Structural Equivalence

| Real Cylinder | Water Container Simulation |
| --- | --- |
| Steel shell: ~15 kg | Heavy object: any known weight |
| Gas: 14.2 kg (BIS fixed) | Water: exactly 14.2 kg = 14.2 litres |
| Gross: ~29 kg | Total: heavy object + water = same |
| gas% = (G - S) / 14.2 * 100 | Identical formula — water removed = gas consumed |

## 9.2 Critical Constraint

Water must be exactly 14.2 kg — weigh on kitchen scale before starting experiment, not measured by volume. Total weight must exceed 26 kg for FRESH_CYLINDER_MIN_KG condition to pass.

## 9.3 Scaled Simulation Option

If 14.2 kg water + 15 kg object is impractical, scale down: use smaller water weight and adjust CAPACITY_KG constant in hub for the experiment only. Change it back after. Physics and logic are identical — only numbers scale.

# 10. Key Engineering Principles — Locked

| Principle | Rule |
| --- | --- |
| No hardcoding | cal_factor and tare always derived from hardware. Never written as a literal constant. |
| No HX711 library | Raw bit-bang only. Three corrupt filters always: LONG_MIN, -1, 0x7FFFFF. |
| No government averages | 473 g/day never used. Every number from this household's actual readings. |
| No gas% without steel | Never compute gross/full_gross*100. Always subtract steel first. |
| No discard history | Store all raw readings. Summary aggregates lose information needed for future intelligence layers. |
| No timestamps on node | ESP32-C3 has no RTC. Hub stamps all readings on receipt with Python datetime. |
| No premature prediction | Burn rate and days_remaining show — until MIN_HOURS_BURNRATE (24h) of real data exists. |
| Graduated quality | Every module returns GOOD / DEGRADED / FAILED. Never just bool. State machine reads quality and routes. |
| Conservative bias | When uncertain, report lower gas estimate. False pessimism causes early ordering. False optimism causes outages. Outages are worse. |
| Honest flagging | ~ prefix on all approximate outputs. Removed automatically when anchor_exact becomes true. Never hide uncertainty. |
| Non-domestic detection | Derived S outside [14.5, 15.8] kg → FLAG and abort. Do not poison config.json. |
| Build order discipline | Verify each layer before building on top of it. No jumping ahead. |

# 11. Hardware Reference — Locked

| Component | Detail |
| --- | --- |
| Node | ESP32-C3 SuperMini — RISC-V, 3.3V GPIO |
| Load cells | 3× YZC-161A 20 kg, parallel wiring on fibre plate |
| ADC | GISLAB HX711 — 3.3V VCC ONLY. Never 5V. |
| GPIO | GPIO4 = DT (DOUT). GPIO3 = SCK. Locked permanently. |
| Hub | Arduino UNO Q AQ3 — arduino@AQ3 (never IP) |
| Transport | BLE only. No WiFi. BlueZ on QRB2210 Linux side. |
| BLE Service UUID | aa206b91-235b-42aa-b370-453a3feedf35 |
| BLE Char UUID | b9b25bb1-f2a9-4545-b48f-295ab2789f41 |
| Device name | GasCylMonitor |
| Payload format | {"grams": float, "quality": "GOOD¦DEGRADED¦FAILED", "sigma": float} |
| Production sketch | 3E004_cal_and_run.ino — node/3E004_cal_and_run/ on AQ3 |
| Arduino IDE | v3.0.7 on Windows. Board: ESP32C3 Dev Module. Port: COM11. USB CDC On Boot: ENABLED. |

End of Session Reference — Gas Cylinder Monitor — 2026-06-16